# CS 350 Final Project

This project involved designing and implementing a smart thermostat prototype using a Raspberry Pi and various components. The prototype demonstrates essential thermostat functionalities, including temperature sensing, user interaction with buttons, visual feedback through LEDs and an LCD display, and simulated server communication via UART. This aligns with SysTec's goal of entering the smart thermostat market with a functional, expandable system.

The thermostat utilizes an AHT20 sensor connected via I2C to continuously monitor ambient temperature. A state machine manages three states: off, heat, and cool. A button on GPIO 24 allows the user to cycle through these states. Indicator LEDs signal the system's status: the red LED pulses when heating is active, and the blue LED pulses when cooling is needed. The temperature set point is adjustable via a button on GPIO 12.

A 16x2 LCD displays the current date, time, temperature, and operational state. Additionally, a status string is sent over UART every 30 seconds to simulate data transmission.

Various hardware architectures were assessed for production, including Raspberry Pi, Microchip microcontrollers, and Freescale (NXP) microcontrollers. While the Raspberry Pi is powerful and supports the project well, its higher cost and power consumption make it less suitable for mass production. Microchip microcontrollers provide a cost-effective, low-power solution, while Freescale microcontrollers offer a balanced performance for embedded applications.

In future development, the thermostat will connect to the cloud via Wi-Fi for data transmission to SysTec’s analytics platform. A Freescale ARM-based solution is preferred for its performance, efficiency, and support for cloud connectivity. This project successfully created a functional thermostat prototype that meets technical and business requirements, laying the groundwork for future enhancements.